Supplier Evaluation and Management Procedure

| Document No. | QMS-PRO-004 | Standard | ISO 9001:2015 Cl. 8.4 |
| --- | --- | --- | --- |
| Revision | 2.0 | Issue Date | [Date] |
| Review Frequency | Annual | Owner | Quality Manager |

# 1. Purpose

To define how [Organisation Name] evaluates, selects, monitors and re-evaluates external providers (suppliers and subcontractors) of products and services that affect product quality. To ensure only capable suppliers are used and that supply chain quality risks are managed.

# 2. Scope

Applies to all external providers of:

- Materials and components incorporated into delivered product.

- Subcontracted processes (e.g. galvanising, machining, NDT, welding).

- Calibration services.

- Inspection and testing services.

- Other services that affect the organisation's ability to deliver conforming product (e.g. transport for sensitive product).

Does not apply to general business services that don't affect product quality (e.g. office cleaning, IT, accounting).

# 3. Definitions

| External provider / Supplier | An organisation that provides products or services to [Organisation Name] that affect product conformance. |
| --- | --- |
| Pre-qualification | The evaluation process completed before a supplier is approved for first use. |
| Re-evaluation | Periodic review of an existing approved supplier's continued capability. |
| Performance monitoring | Ongoing tracking of supplier performance against defined criteria. |
| Critical supplier | A supplier whose product or service directly affects product conformance or safety. |
| Approved Supplier Register | QMS-REG-SUP-001 — the controlled list of pre-qualified suppliers. |

# 4. Supplier Categorisation

| Category | Definition | Examples |
| --- | --- | --- |
| Critical | Supply directly affects product conformance or safety. Failure has significant cost, schedule or quality impact. | Structural steel suppliers; certified welding subcontractors; NDT providers; calibration providers. |
| Standard | Supply contributes to product conformance but is not on the critical path. Failure has manageable impact. | Consumables (welding rods, paints), site services, freight providers, hardware suppliers. |
| Non-critical | Supply does not directly affect product conformance. | Stationery, IT, catering, general consumables. |

Categorisation drives the level of evaluation, monitoring and re-evaluation applied. Critical suppliers receive the most rigour.

# 5. Pre-Qualification

Before any new Critical or Standard supplier is used, the Quality Manager (or delegate) completes a Pre-Qualification Assessment covering:

- Quality system —  ISO 9001 certification (preferred for Critical) or equivalent QMS evidence.

- Technical capability —  Equipment, qualifications, scope of accreditation, past project evidence.

- Financial stability —  Credit check, trading history, ABN verification.

- Insurance —  Public liability (minimum $10M), professional indemnity (where applicable), workers compensation current.

- Safety performance —  LTIFR, WHS management system, incident history (for subcontractors).

- References —  At least two customer references for Critical suppliers.

- Site audit —  Where supply is Critical and contract value or volume justifies, an on-site audit of the supplier's facility.

The Pre-Qualification record is filed and the supplier added to QMS-REG-SUP-001 with initial rating, scope of supply and review date.

# 6. Approval

Supplier approval authority:

- Non-critical suppliers —  Approved by Procurement.

- Standard suppliers —  Approved by Quality Manager.

- Critical suppliers —  Approved jointly by Quality Manager and Operations Manager.

- Critical suppliers  **>**** $250K annual spend — **Approved by Managing Director.

Approval is recorded in QMS-REG-SUP-001 with date, approver and scope of supply.

# 7. Communication of Requirements (Cl. 8.4.3)

Before each purchase or subcontract, the requirements communicated to the supplier must include:

- Description of the products or services, including specifications and acceptance criteria.

- Approval and release requirements (e.g. customer hold points, inspection requirements).

- Competence requirements for supplier personnel where relevant (e.g. welder qualifications, NDT certifications).

- Interactions and the supplier's role within the organisation's QMS.

- Inspection and verification activities [Organisation Name] or its customer intend to perform at the supplier's premises.

- Statutory and regulatory requirements applicable to the supply.

- Records the supplier must produce and provide (mill certs, NDT reports, COCs).

These requirements are flowed down through QMS-PRO-001 §5.6.

# 8. Performance Monitoring

Supplier performance is monitored against the following criteria, recorded in QMS-REG-SUP-001:

- On-time delivery (OTD) —  Target ≥ 95% for Critical, ≥ 90% for Standard.

- Quality conformance —  Incoming nonconformance rate. Target < 2% for Critical, < 5% for Standard.

- Documentation —  Completeness and timeliness of supplied certificates and records.

- Responsiveness —  Issue resolution speed; willingness to engage with corrective action.

- Safety —  For subcontractors: incident-free engagements, induction compliance.

Performance data is collected from QA inspection records, receipt records, project sign-offs and CAR records.

# 9. Re-Evaluation Schedule

- Critical suppliers:  Re-evaluated at least annually.

- Standard suppliers:  Re-evaluated at least every 24 months.

- Non-critical suppliers:  Re-evaluated on performance event only.

- Re-evaluations are triggered earlier if performance deteriorates.

- Re-evaluation outcomes update the supplier's rating in QMS-REG-SUP-001.

# 10. Out-of-Conformance Response

- Rating C (Conditional):  Formal improvement plan issued. Monthly performance monitoring for 90 days. Re-evaluation at end of period.

- Rating D (Suspended):  No new purchase orders. Existing orders assessed for risk. Quality Manager notifies supplier in writing.

- Major NC from a supplier:  Triggers a Supplier CAR (QMS-FRM-CAR-001). Supplier required to provide root cause and corrective action within 30 days.

- Repeated NC from a supplier:  Triggers rating review and possible suspension regardless of current rating.

# 11. Records

| Pre-qualification records | Retained while supplier active + 3 years. |
| --- | --- |
| Approved Supplier Register | Maintained current (QMS-REG-SUP-001). |
| Performance evaluation records | Retained 5 years. |
| Supplier CARs | Retained 7 years — linked to NCR register. |
| Insurance and certification copies | Updated annually. Retained while supplier active + 3 years. |
| Purchase orders and subcontracts | Retained in project file 7 years minimum. |

# 12. Related Documents

| Document No. | Title |
| --- | --- |
| QMS-PRO-001 | Customer Requirements & Contract Review |
| QMS-PRO-003 | Control of Nonconforming Outputs |
| QMS-PRO-005 | Product and Service Release |
| QMS-PRO-009 | Corrective Action Procedure |
| QMS-REG-SUP-001 | Approved Supplier Register |

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Quality Manager |  |  |  |
| Procurement Manager |  |  |  |
| Managing Director |  |  |  |

✎ HOW TO COMPLETE THIS DOCUMENT — DELETE THIS SECTION BEFORE PUBLISHING

Use these notes to tailor, complete and finalise this document before publication. Delete this section once done.

1. Categorise every current supplier as Critical, Standard or Non-critical Walk the AP supplier list with the Quality Manager and Procurement Manager. Categorise based on Section 4. Update QMS-REG-SUP-001.

2. Build the Pre-Qualification Assessment form Section 5 lists the criteria. Build a simple form (Word, PDF or portal workflow) that captures evidence against each criterion.

3. Audit the current supplier base Identify any current Critical or Standard supplier without complete pre-qualification evidence. Either qualify or stop using.

4. Set up performance monitoring data feeds Where is OTD data coming from? Where is incoming inspection conformance data coming from? Confirm the data sources before relying on them.

5. Build the Supplier CAR workflow When a Major NC is attributed to a supplier, an external CAR must go out. Build the template.

6. Schedule the re-evaluation cycle Diarise re-evaluation dates per Section 9. Critical suppliers annually, Standard every 24 months.

7. DELETE this guidance section before publishing Select the entire yellow-bordered block and delete.

QMS-PRO-004  •  Rev 2.0	CONTROLLED DOCUMENT — verify currency before use	Page  of